Task 5 – Hands-On Lab

MQTT-Based Temperature & Humidity Data Pipeline using M5Core2 and Raspberry Pi

Objective

The objective of this lab is to demonstrate an end-to-end IoT data pipeline using the MQTT protocol. In this task, an M5Stack Core2 device publishes temperature and humidity data to an MQTT broker running on a Raspberry Pi Zero W2. A Python MQTT client subscribes to the data, logs it into a CSV file, and visualizes the results.

Learning Outcomes

After completing this lab, students will be able to:

- Understand the MQTT publish/subscribe communication model

- Configure an MQTT broker using Mosquitto on Raspberry Pi

- Program the M5Core2 to publish sensor data via Wi-Fi

- Develop a Python MQTT subscriber

- Store and visualize IoT data

Hardware Requirements

- M5Stack Core2

- Raspberry Pi Zero W2

- Micro-USB / USB-C cable

- Laptop (Windows/Linux/macOS)

- Wi-Fi network (same network for Pi and M5Core2)

Software Requirements

- Arduino IDE

- ESP32 Arduino Core (version 2.0.11)

- M5Core2 Arduino Library

- PubSubClient Arduino Library

- Raspberry Pi OS (Headless)

- Mosquitto MQTT Broker

- Python 3

- Python libraries: paho-mqtt, matplotlib

## System Architecture

- M5Core2  →  Wi-Fi  →  MQTT Broker (Raspberry Pi)  →  Python Subscriber  →  CSV + Plot

## TASK 1: AHT20+M5Core2 > RPI ZeroW2 (T & H)

M5Core2_AHT20_MQTT.ino > mqtt_loc_consumer.py

## Part A: MQTT Broker Setup on Raspberry Pi

### Step 1: Update Raspberry Pi

- sudo apt update

- sudo apt upgrade -y

### Step 2: Install Mosquitto

- sudo apt install mosquitto mosquitto-clients -y

Enable and start the broker:

- sudo systemctl enable mosquitto

- sudo systemctl start mosquitto

Verify:

- systemctl status mosquitto

### Step 3: Enable Remote MQTT Access

- Edit the configuration file:

- sudo nano /etc/mosquitto/mosquitto.conf

- Add the following lines at the end:

- listener 1883

- allow_anonymous true

- Save and exit, then restart Mosquitto:

- sudo systemctl restart mosquitto

Verify broker is listening:

- ss -tulpn | grep 1883

### Step 4: Test MQTT Broker

- Open two terminals on the Raspberry Pi.

- Terminal 1 (Subscriber):

- mosquitto_sub -h localhost -t lab/temphum

- Terminal 2 (Publisher):

- mosquitto_pub -h localhost -t lab/temphum -m '{"temp":25.5,"hum":60.2}'

- The message should appear in Terminal 1.

## Part B: M5Core2 MQTT Publisher

Step 5: Arduino IDE Configuration

- Board: M5Stack-Core2

- Port: Select correct COM port

- M5Core Board version: 3.2.5

Step 6: M5Core2 Firmware

Upload a sketch that:

- Connects to Wi-Fi

- Connects to Raspberry Pi MQTT broker

- Publishes JSON messages to topic lab/temphum

Example payload:

{"temp":22.8,"hum":50.0}

### Connection Diagram: AHT20 to M5Core2 (Port A)

The M5Core2 uses Port A (the Red Grove port usually located on the side of the device) for I2C connections. The internal mapping for this port is SDA = GPIO 32 and SCL = GPIO 33.

#### Wiring Table

If you are using a standard Grove Cable (M5Stack Unit):

- Simply plug the 4-pin connector into the Red Port A on the M5Core2.

| M5Core2 (Port A) | Wire Color (Standard Grove) | AHT20 Sensor Pin | Description |
| --- | --- | --- | --- |
| GND | Black | GND | Ground |
| 5V | Red | VIN / VCC | Power (2.0 - 5.5V) |
| G32 | White | SDA | I2C Data |
| G33 | Yellow | SCL | I2C Clock |

Install Arduino Library

  Adafruit AHTX0 (by Adafruit)

  PubSubClient (by Nick O'Leary)

  ArduinoJson (by Benoit Blanchon)

Step 7: Verify M5Core2 Output

Open Serial Monitor (115200 baud) and verify:

- Wi-Fi connected

- MQTT connected

- JSON messages printed periodically

## Part C: Python MQTT Subscriber on Raspberry Pi

Step 8: Python Environment Setup

cd day8

python3 -m venv venv

source venv/bin/activate

pip install paho-mqtt matplotlib

Step 9: Run MQTT Consumer

python mqtt_consumer.py

Expected output:

temp=22.8 hum=50.0

Step 10: CSV Data Logging

Verify CSV file:

ls -lh temphum_log.csv

head temphum_log.csv

The file should contain timestamped temperature and humidity values.

TASK 2: M5Core2 using Unified library to interface the AHT20 sensor to transmit MQTT data of Location, temperature and humidity. Average received data from single end-device

Arduino/M5Core2: No Changes

RPi Zero W2 (local master): Run mqtt_consumer_avg.py

TASK 3: RPI with PCT2075

M5Core2 using Unified library to interface the AHT20 sensor to transmit MQTT data of Location, temperature and humidity. Average received data from multiple end-device, and local master on the global master.

PCT2075 is a temperature-only sensor, the code includes a placeholder None (null) for humidity to ensure the data structure remains compatible with your Master Node's expectation of "Location, Temperature, and Humidity."

### Prerequisites

- Enable I2C: Run sudo raspi-config → Interface Options → I2C → Yes.

- Install Libraries: You will need the Adafruit CircuitPython driver and the MQTT client.

pip3 install adafruit-circuitpython-pct2075 paho-mqtt

| Raspberry Pi Zero W2 Pin | PCT2075 Pin | Function |
| --- | --- | --- |
| Pin 1 (3.3V) | VCC / VIN | Power (2.7V - 5.5V) |
| Pin 3 (GPIO 2) | SDA | I2C Data Line |
| Pin 5 (GPIO 3) | SCL | I2C Clock Line |
| Pin 6 (GND) | GND | Ground |

Arduino/M5Core2: (change mqtt address)

RPi Zero W2: Run rpi_publisher_pct2075.py (need to change the mqtt address)